Reading measured value block

Display group overview

| Display group no. | Indicated on display | Designation |  |  |
| --- | --- | --- | --- | --- |
| 001 | Read measured value block 1 | 1 | = | Engine speed |
|  | 1 2 3 4 | 2 | = | Intake manifold pressure |
|  |  | 3 | = | Ignition timing |
|  |  | 4 | = | Position of idling stabilisation valve |
| 002 | Read measured value block 2 | 1 | = | Throttle valve angle |
|  | 1 2 3 4 | 2 | = | Intake manifold pressure |
|  |  | 3 | = | Coolant temperature |
|  |  | 4 | = | Intake air temperature |
| 003 | Read measured value block 3 | 1 | = | Lambda learning value (bank 1) |
|  | 1 2 3 4 | 2 | = | Lambda learning value (bank 2) |
|  |  | 3 | = | Lambda control (bank 1) |
|  |  | 4 | = | Lambda control (bank 2) |

| Display group no. | Indicated on display | Designation |  |  |
| --- | --- | --- | --- | --- |
| 004 | Read measured value block 4 | 1 | = | Condition of lambda control |
|  |  | 2 | = | Tank venting learning value |
|  |  | 3 | = | Lambda control (bank 1) |
|  |  | 4 | = | Lambda control (bank 2) |
| 005 | Read measured value block 5 | 1 | = | Engine speed |
|  |  | 2 | = | Knock control depth |
|  |  | 3 | = | Ignition timing |
|  |  | 4 | = | Coolant temperature |
| 006 | Read measured value block 6 | 1 | = | Position of idling stabilisation valve |
|  | 1 2 3 4 | 2 | = | Learned value for idling stabilisation |
|  |  | 3 | = | Idling stabilisation valve interference level |
|  |  | 4 | = | Idling stabilisation valve control |
| 007 | Read measured value block 7 | 1 | = | Position of idling stabilisation valve |
|  | 1 2 3 4 | 2 | = | Coolant temperature |
|  |  | 3 | = | Lambda learning value (bank 1) |
|  |  | 4 | = | Lambda learning value (bank 2) |

| Display group no. | Indicated on display | Designation |  |  |
| --- | --- | --- | --- | --- |
| 008 | Read measured value block 8 | 1 | = | Position of idling stabilisation valve |
|  | 1 2 3 4 | 2 | = | Coolant temperature |
|  |  | 3 | = | Lambda learning value (bank 1) |
|  |  | 4 | = | Lambda learning value (bank 2) |
| 009 | Read measured value block 9 | 1 | = | Coolant temperature |
|  | 1 2 3 4 | 2 | = | Idling switch |
|  |  | 3 | = | Ignition timing |
|  |  | 4 | = | Engine speed |
| 010 | Read measured value block 10 | 1 | = | Coolant temperature |
|  | 1 2 3 4 | 2 | = | Idling switch |
|  |  | 3 | = | Lambda learning value (bank 1) |
|  |  | 4 | = | Tank venting learning value |
| 099 | Read measured value block 99 | 1 | = | Engine speed |
|  | 1 2 3 4 | 2 | = | Engine load |
|  |  | 3 | = | Coolant temperature |
|  |  | 4 | = | Lambda control OFF/ON |

Display group 001

|  | Reading measured value block 1 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... rpm | ... % | ... oBTDC | ... |  |  |  |
|  |  |  |  |  | Position of idling stabilisation valve ▪ 18 ... 75 OK |  |  |
|  |  |  |  | Ignition firing point - ignore |  |  |  |
|  |  |  | Intake manifold pressure (engine load) ▪ 29 ... 59% OK (100% = 1022 hPa, 32% = 327 hPa) |  |  |  |  |
|  |  | Engine speed (idling speed) ▪ 680 ... 820 rpm OK |  |  |  |  |  |

Test table, Display group 001

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 | Greater than 820 rpm | - Idling switch -F60 defective or incorrectly set | - Check and adjust idling speed switch -F60 => Page 24-99 or => Display group 009, Display zone 2 |
|  |  | - Air intake system drawing in large quantities - Unmetered air | - Check air intake system for leaks |
|  |  | - Idling stabilisation valve -N71 is sticking or stiff | - Check idling stabilisation valve -N71 => Page 24-55. |
|  | Less than 680 rpm | - Idling stabilisation valve -N71 is sticking or stiff | - Check idling stabilisation valve -N71 => Page 24-55. |
|  |  | - Idling switch -F60 defective or incorrectly set | - Check and adjust idling speed switch -F60 => Page 24-99 or => Display group 009, Display zone 2 |
|  |  | - Loads switched on, e.g. air conditioner is on; steering wheel is at end stop; selector lever of automatic gearbox not set to position "P" or "N" | - Switch off all electrical loads Centre steering wheel Move selector lever to "P" or "N" |

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 2 | Outside the tolerance range | - Poor idling (not running on all cylinders) | - Injector or spark plugs defective |
|  |  | - Idling stabilisation valve -N71 defective | - Interrogate fault memory =>Page 01-10 |
|  |  | - Loads switched on, e.g. air conditioner is on; steering wheel is at end stop; selector lever of automatic gearbox not set to position "P" or "N" | - Switch off all electrical loads Centre steering wheel Move selector lever to "P" or "N" |
| 3 | ... | ignore |  |
| 4 | Outside the tolerance range | - Idling stabilisation valve -N71 is sticking or stiff | - Check idling stabilisation valve -N71 => Page 24-55. |

Display group 002

|  | Reading measured value block 2 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... % | ... % | ... oC | ... oC |  |  |  |
|  |  |  |  |  | Intake air temperature ▪ No specified value - depends on ambient temperature |  |  |
|  |  |  |  | Coolant temperature ▪ 81 ... 111 oC |  |  |  |
|  |  |  | Intake manifold pressure => Display group 001, display zone 2 |  |  |  |  |
|  |  | Throttle valve angle: ▪ Idling:0% OK ▪ Full load:95% OK |  |  |  |  |  |

Test table, Display group 002

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 | Greater than 0% at idling speed | - G69 throttle valve potentiometer defective | - Interrogate fault memory =>Page 01-10 |
|  |  | - Throttle cable setting | - Adjust throttle cable |
|  |  | - Throttle valve sticking | - Eliminate cause |
|  | Less than 95% at full throttle | - G69 throttle valve potentiometer defective | - Interrogate fault memory =>Page 01-10 |
|  |  | - Throttle cable setting | - Adjust throttle cable |
|  |  | - Throttle valve potentiometer -G69 incorrectly set | - Check throttle valve potentiometer -G69 => Page24-94 |

Note on display zone 1:

For checking at full throttle, switch engine off and ignition on.

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 3 | Less than | - Engine too cold | - Perform test drive if necessary |
|  | 81 oC 1) | - Coolant temperature sensor or wiring to engine control unit | - Interrogate fault memory =>Page 28-32 |
|  | Greater than | - Radiator dirty | - Clean radiator |
|  | 111 oC 1) | - Radiator fan not working | - Check function |
|  |  | - Thermostat defective | - Check coolant thermostat |
|  |  | - Coolant temperature sensor or wiring to engine control unit | - Interrogate fault memory =>Page 28-32 |
| 4 | Differs from ambient temperature | - Intake air temperature sensor -G42 defective | - Interrogate fault memory =>Page 28-27 |

1)    Vehicle at operating temperature.

Note on display zone 3:

The coolant temperature sensor is a temperature-sensitive resistor. If the sensor signal is biased, for example due to moisture in a connector (effect similar to parallel resistor), this biasing may be in a range which the fault memory is still not able to detect.

Display group 003

|  | Reading measured value block 3 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | ... | ... | ... |  |  |  |
|  |  |  |  |  | Lambda control (bank 2) ▪ 0.75 ... 1.25 OK |  |  |
|  |  |  |  | Lambda control (bank 1) ▪ 0.75 ... 1.25 OK |  |  |  |
|  |  |  | Lambda learning value (bank 2) ▪ 0.75 ... 1.25 OK |  |  |  |  |
|  |  | Lambda learning value (bank 1) ▪ 0.75 ... 1.25 OK |  |  |  |  |  |

Test table, Display group 003

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 / 2 | Low | - Fuel pressure too high | - Check fuel pressure =>Page 24-24. |
|  | Lambda | - Injector leaking | - Check injection valves => Page 24-30 |
|  | learned values | - Solenoid valve 1 for activated charcoal filter -N80 is constantly open | - -Check N80 => Page 24-86 |
|  |  | - Lambda probe heater defective or Lambda probe dirty | - Checking Lambda probe => Page 24-65 |
|  | high lambda | - Injector blocked | - Test injection quantity =>Page 24-41. |
|  | learned values | - Fuel pressure too low | - Check fuel pressure =>Page 24-24. |
|  |  | - Unmetered air at exhaust manifold gasket | - Eliminate cause |
|  |  | - Lambda probe heater defective or Lambda probe dirty | - Checking Lambda probe => Page 24-65 |

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |  |
| --- | --- | --- | --- | --- |
| 3 / 4 | Outside tolerance range | Small value - engine too rich Effect:: Lambda control leans down the mixture | - Wait 30 seconds for the display to stabilise |  |
|  |  | Large value - engine too lean Effect:: Lambda control enriches the mixture |  |  |
|  |  | - Unmetered air | - Eliminate source of unmetered air |  |
|  |  | - Injector defective | - Check injection quantity =>Page 24-41. |  |
|  |  | - Not successfully learned | - => Display zones 1 and 2 |  |
|  |  | - Learned Lambda value at limit | - Check Lambda readiness to learn => Display group 004 |  |

Display group 004

|  | Reading measured value block 4 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | ... | ... | ... |  |  |  |
|  |  |  |  |  | Lambda control (bank 2) => Display group 003, display zone 4 |  |  |
|  |  |  |  | Lambda control (bank 1) => Display group 003, display zone 3 |  |  |  |
|  |  |  | Tank venting learning value ▪ 0.75 ... 1.25 OK |  |  |  |  |
|  |  | Lambda control status: ▪ 0: Lambda control learning function active (ACF valve not actuated) ▪ 16: Lambda control learning function not active (ACF valve actuated) |  |  |  |  |  |

Note on display zone 1:

A value of 0 must be displayed in display zone 1 for 1 minute after starting up (ACF valve not active), then a value of 16 for 7 minutes (ACF valve active) and then a value of 0 must be displayed again for 1 minute. A value between 1 and 12 is displayed if there is a fault in the system => Checking table Page 01-74.

Test table, Display group 004

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 | 1 | - Coolant temperature too low | - Coolant temperature of 85 oC not yet reached Interrogate fault memory =>Page 28-32 |
|  | 4 | - Idling speed too low | - Switch off all electrical loads Interrogate fault memory =>Page 24-20 |
|  | 5 | - Idling speed too high | - Interrogate fault memory =>Page 24-20 |
| Cont.- ▼ | 6 | - Lambda probe not ready | - Interrogate fault memory =>Page 24-81 |

| Display zone | Display | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 | 7 | - Lambda control inhibited | - Coolant temperature of 85 oC not yet reached Interrogate fault memory =>Page 24-94 |
|  | 10 | - Learning value not attained | - Coolant temperature of 85 oC not yet reached Interrogate fault memory =>Page 01-10 |
|  | 11 | - Too few firing points per evaluation interval | - Interrogate fault memory =>Page 01-10 Loose contact in Lambda probe wiring |
|  | 12 | - Too many firing points per evaluation interval | - Interrogate fault memory =>Page 01-10 Lambda probe aged |
| 2 | lower than 0.75 | - Activated charcoal filter system solenoid valve sticking | - Test ACF solenoid valve =>Page 24-86 |
|  | higher than 1.25 | - ACF-system open | - Check ACF system hose connections |

Display group 005

|  | Reading measured value block 5 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ...rpm | ...oATDC | ...oBTDC. | ... oC |  |  |  |
|  |  |  |  |  | Coolant temperature => Display group 002, display zone 3 |  |  |
|  |  |  |  | Average of all ignition timing angles |  |  |  |
|  |  |  | Knock control depth ▪ Basic value: 60o ATDC (when the ignition timing is retarded by the knock control the amount of retardation is subtracted from the 60o ATDC) |  |  |  |  |
|  |  | Engine speed =>Display group 001, display zone 1 |  |  |  |  |  |

Test table, Display group 005

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 2 | Less than 60o | - Abnormal engine running noise (ancillaries loose) - Poor quality fuel (less than RON 91) - Intake air temperature too high | - Rectify cause |

Display group 006

|  | Reading measured value block 6 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | ... | ... | ... |  |  |  |
|  |  |  |  |  | Idling speed stabilisation control - ignore |  |  |
|  |  |  |  | Idling speed stabilisation interference level ▪ 0 = no interference |  |  |  |
|  |  |  | Idling stabilisation valve learning value ▪ 18 ... 75 OK (pre-control deviations from basic value in accordance with engine and operating conditions) |  |  |  |  |
|  |  | Position of idling stabilisation valve ▪ 10 ... 60 OK |  |  |  |  |  |

Test table, Display group 006

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 1 | lower than 10 | - Intake air system drawing in unmetered air | - Check air intake system for leaks |
|  | higher than 60 | - Idling stabilisation valve -N71 defective | - Interrogate fault memory =>Page 24-55. |
| 3 | higher than 5 | - Automatic gearbox selection lever not in position "P" or "N" | - Move selector lever to "P" or "N" |
|  | higher than 10 | - Air conditioner switched on | - Switch off the air conditioner |

Display groups 007 and 008:

|  | Read measured value block 7/8 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | ... oC | ... | ... |  |  |  |
|  |  |  |  |  | Lambda control (bank 2) => Display group 003, display zone 4 |  |  |
|  |  |  |  | Lambda control (bank 1) => Display group 003, display zone 3 |  |  |  |
|  |  |  | Coolant temperature => Display group 002, display zone 3 |  |  |  |  |
|  |  | Idling speed stabilisation valve position => display group 006, display zone 1 |  |  |  |  |  |

Display Group 009:

|  | Reading measured value block 9 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... oC | ... | ...oBTDC. | ...rpm |  |  |  |
|  |  |  |  |  | Engine speed => Display group 001, display zone 1 |  |  |
|  |  |  |  | Ignition firing point - ignore |  |  |  |
|  |  |  | Idling switch ▪ 0 = open ▪ 1 = closed |  |  |  |  |
|  |  | Coolant temperature => Display group 002, display zone 3 |  |  |  |  |  |

Test table, Display group 009

| Display zone | Readout on V.A.G 1551 | Cause of fault | Fault remedy |
| --- | --- | --- | --- |
| 2 | 0 or 1 | - F60 idling switch defective - Throttle valve potentiometer -G69 defective or incorrectly set | - Interrogate fault memory =>Page 24-99 |
|  |  | - Throttle valve sticking - Floor mat pressing down on accelerator pedal | - Eliminate cause |
|  |  | - Moisture in connector of throttle valve potentiometer -G69 | - Check connector |
|  |  | - Throttle cable setting | - Adjust throttle cable |

Display Group 010:

|  | Reading measured value block 10 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... oC | ... | ... | ...rpm |  |  |  |
|  |  |  |  |  | Tank venting learned value => Display group 004, Display zone 2 |  |  |
|  |  |  |  | Lambda control (bank 1) => Display group 003, display zone 3 |  |  |  |
|  |  |  | Idling speed switch => Display group 009, Display zone 2 |  |  |  |  |
|  |  | Coolant temperature => Display group 002, display zone 3 |  |  |  |  |  |

Display group 099, Lambda control:

|  | Reading measured value block 99 |  |  | ⇒ | ◂ Indicated on display |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ...rpm | ...% | ... oC | λ-control... |  |  |  |
|  |  |  |  |  | Lambda control ▪ Basic setting 04: λ-control OFF ▪ Reading measured value block 08: λ-control ON |  |  |
|  |  |  |  | Coolant temperature => Display group 002, display zone 3 |  |  |  |
|  |  |  | Engine load => Display group 001, display zone 2 |  |  |  |  |
|  |  | Engine speed =>Display group 001, display zone 1 |  |  |  |  |  |

Notes on display zone 4:

- ◆ For defined fault finding, the lambda control is switched off when display group 099 in function 04 "Basic setting" is selected and is switched on when this group is selected in function 08 "Reading measured value block".

- ◆ Lambda control is automatically re-activated on exit from function 04 "Basic setting".