250W-ATX

Installation Guide

PC Motherboard Power ON/OFF

A . ATX Power Output Connector 20/24 pin Connector

A1.. 12V-ATX power output connector 4 pin

C. DC V-

D.GND

E. DC V+

ITPS Mode

Power Output Connectors

E :Battery + (un-switched battery, positive)

D : GND

C : Battery -(negative)

Controls and Settings

B /B1 : To Motherboard ON/OFF switch

J10 : User jumper settings (1,2,3)

Power Output Connectors

A 1:  Optional P4-12V power

A :  ATX Power Output Connector 20/24 pin connector

NOTE: “If HARDOFF is set to “never”, 250W_ATX will

automatically shut down when battery voltage is below

11.2V for more than 1 minute in order to prevent ‘deep

discharge’ situations.

Mode “0” is regular ATX power supply mode, no power

sequencing provided, can be used for non vehicle

applications.

Avoid using HARDOFF = Never, can severely discharge

your battery if PC. Suggested modes are: 1, 2, and 4.

1.2 Power challenges in a Vehicle PC

The 5V Standby Problem: One of most difficult tasks of

operating a PC in a vehicle is power consumption while the

computer is OFF. Even when your computer is OFF, it will still

consume about 100mA on the 5V rail. All power supplies

provide 5VSB (5V standby) so that the motherboard can issue

at least a PSON signal. When the computer is in the suspend

mode, it will consume even more power, because the RAM

needs to be powered at all times.

No matter how big your battery is, it will eventually drain your

battery in a matter of days.

| DC/DC ATX power supply for automobile application, 8--28VDC input. |  |  |  |  |
| --- | --- | --- | --- | --- |
| +5V,+12V,+3.3V,-12V,+5VSB and PG (power good signal) outputs. |  |  |  |  |
| Size |  |  |  |  |
| 1.The smallest 8--28V DC to DC power supply |  |  |  |  |
| 2. convertor in the world. |  |  |  |  |
| 3.Adopt industrial solid capacitor, ideal for being used in |  |  |  |  |
| 4. harsh evironment |  |  |  |  |
| 5.High performance with high heat-stand feature, more realiable |  |  |  |  |
| 6.Fanless DC-DC solution for 100% mute operating . |  |  |  |  |
| 7.Compact design, special for mini-itx and 1U case. |  |  |  |  |
| Output Voltage Regulation |  |  |  |  |
| Output | Min Load(A) | MAX Load(A) | PEAK Load(A) | Regulation |
| +12V | 0 | 10 | 12 | ±5% |
| +5V | 0 | 10 | 12 | ±5% |
| +3.3V | 0 | 10 | 12 | ±5% |
| -12V | 0 | 0.1 | 0.1 | ±15% |
| +5VSB | 0 | 2 | 2 | ±5% |

| : |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1.Dimensions(mm): L120x W95x20H |  |  |  |  |
| 2.DC Input: 8--28V |  |  |  |  |
| 3.DC Output : 210W |  |  |  |  |
| 4.Input Current: Max. 15A |  |  |  |  |
| 5.Efficiency: above 92% |  |  |  |  |
| 6.MTPF: 90,000H |  |  |  |  |
| 7.Operating Temperature: 0℃~ 75℃ |  |  |  |  |
| 8.Storage Temperature: -40℃~ 70℃ |  |  |  |  |